# Automotive Door Control System

# Dynamic Design

Owner: Mohamed Hossam , Email: mohamed.hossam.1183@gmail.com

# ECU_1

## ECU State Machine Diagram

## ECU Sequence Diagram

See “Raw_Diagrams” folder for high resolution image.

## CPU Load

- WCET (worst case execution time) analysis:
utilization factor of one frame = execution time * frequency = execution time / Period
CPU Load = summation of utilization factor for all tasks

- Door state message will be sent by ECU_1 every 10 ms to ECU_2.

- Light switch state message will be sent by ECU_1 every 20 ms to ECU_2.

- Speed state message will be sent by ECU_1 every 5 ms to ECU_2.

- In ECU 1, assuming tasks required to periodically send CAN status frames are identical,
& also assuming to have an execution time of (1 ms).

- CPU Load for ECU 1 =
( 1 / 10 ) + ( 1 / 20 ) + ( 1 / 5 ) = 0.35 = 35 %

## ECU Components State Machine Diagrams

- ( OS - BCM - VIM - DS_drv - LS_drv - SS_drv - GPIO - CAN_If - CAN_drv )

Diagrams collected inside “ECU_1_Components_State_Diagrams” folder.

# ECU_2

## ECU State Machine

## ECU Sequence Diagram

See “Raw_Diagrams” folder for high resolution image.

## CPU Load

- WCET (worst case execution time) analysis:
utilization factor of one frame = execution time * frequency = execution time / Period
CPU Load = summation of utilization factor for all tasks

- In ECU 2, assuming tasks required to periodically receive CAN status frames are identical,
& also assuming to have an execution time of (1 ms).

- ECU_2 has a periodic task to call the "VIO_Main" API to evaluate sensor values and operate warnings accordingly.
This task is called every (5 ms)

- And Assuming the task, required to call the "VIO_Main" API, has an execution time of (2 ms).

- CPU Load for ECU 2 =
( 1 / 10 ) + ( 1 / 20 ) + ( 1 / 5 ) + ( 2 / 5 ) = 0.75 = 75 %

## ECU Components State Machine Diagrams

- ( OS - BCM - VIO - Bz_drv - Li_drv - GPIO - CAN_If - CAN_drv )

Diagrams collected inside “ECU_2_Components_State_Diagrams” folder.

## VIO_Main function Flowchart

Periodically evaluate sensor values and operate warnings accordingly.

# CAN Bus Load

- Regarding CAN bus load calculation, assuming “standard CAN” frame consist of below fields:

- Start-of-frame (1 bits): Denotes the start of frame transmission

- Identifier (11 bits): unique identifier, also represents message priority

- Remote transmission request (RTR) (1 bits): dominant (0) for data frames, and recessive (1) for request frames.

- Identifier extension bit (IDE) (1 bits): It indicates standard CAN frame is being transmitted with no extension.

- Reserved bit (r0) (1 bits): Must be dominant (0).

- Data length code (DLC) (4 bits):  Number of bytes of data (0–8 bytes)

- Data field (red) (0-64 bits): Data to be transmitted.

- CRC (15 bits): Cyclic redundancy check

- Bit stuffing is possible in some of the above fields around (18 bits) in the worst case.

- CRC delimiter (1 bits): Must be recessive (1)

- ACK slot (1 bits): Transmitter sends recessive (1) and any receiver can assert a dominant (0)

- ACK delimiter (1 bits): Must be recessive (1)

- End-of-frame (EOF) (7 bits): Must be recessive (1)

- Inter-frame spacing (IFS) (3 bits): Must be recessive (1)

- So 1 CAN frame contains approximately 125 bit.

Assuming we are using 500 kBit/s bit rate.

bit time = 1 / bit rate = 1 / (500 * 1000) s = 0.002 ms

This means 1 bit will take 0.002 ms to transfer on bus when using 500 kBit/s.

So the time to transfer 1 frame carrying 125 bits is (0.002 ms/bit * 125 bit) = 0.25 ms.

- Door state message will be sent every 10ms.
Light switch state message will be sent every 20ms.
Speed state message will be sent every 5ms.

Assuming all 3 frames are identical
i.e same transmission time (0.25 ms).

- WCET (worst case execution time) analysis:
utilization factor of one frame = transmission time * frequency = transmission time / Period

- Bus Load = summation of utilization factor for all frames =
( 0.25 / 10 ) + ( 0.25 / 20 ) + ( 0.25 / 5 ) = 0.0875 = 8.75 %